VarioUltra Quick Reference Guide:
Welcome to the VarioUltra Quick Reference Guide. The purpose of this document is to introduce you to the primary functions of the device and let you know where to find additional resources. An electronic copy of this guide and the user manual are available at our web site, and on the VarioUltra

VarioUltra Email:

The VarioUltra email list is a way for customers, prospective customers, and members of the VisioBraille support team to interact. Whether you need technical support outside of regular office hours, or just want to ask or respond to a question, the VarioUltra list is for you. To subscribe, send an email to varioultra-request@freelists.org. Once you are subscribed, you can send a message to the list by emailing to varioultra@freelists.org.

What’s in the Box?

Besides this quick start guide, the VarioUltra box includes the following items: a VarioUltra 20 or VarioUltra 40, a carry case and strap, a micro USB cable with assorted plug adaptors, and a print user manual. Please keep the box in case you ever need to ship VarioUltra to us for service or Upgrade.

VarioUltra Orientation:
Topside. The top of VarioUltra contains an 8-button braille keyboard, beneath which you will find a braille display with cursor routing buttons above each cell. There are display keys left and right of the display. Display keys d1, d2, and d3 on the left and d4, d5, and d6 on the right. The angled front of VarioUltra features 7 buttons. From the left you will find system keys s1 and s2, the left thumb key b9, the 5-way Navistick, the right thumb key b0, and system keys s3 and s4.
Left side. From the left rear corner of VarioUltra you will find: Standard USB Type A interface for connecting a USB memory stick, a micro USB interface for connecting to a USB power supply, or for connecting to a computer, a device mode switch that slides away from you for braille notetaker mode and toward you for braille display mode, and a device lock switch that slides away from you to unlock and toward you to lock VarioUltra.
Right side. The right side of VarioUltra is where you will find the power button. A short press toggles VarioUltra on and off. Press and hold for 10 seconds to reboot. Hold down the system key 4 and press the power button to power VarioUltra off. If VarioUltra is in Braille Display mode and is connected to a Bluetooth device, the connection will be lost. If VarioUltra is in Braille Display mode and is not actively connected to an external device, or if VarioUltra is in Notetaker mode and no application is being used, VarioUltra goes into standby mode automatically after 10 minutes of inactivity.

Getting Started:
Note that all VarioUltra units are shipped in the locked position. You will find the unlock slider switch on the left side of VarioUltra. It needs to be facing toward the back of the device.
If VarioUltra is in standby mode, pressing the power button will wake it up at the last place you were working. If VarioUltra was off completely, pressing the power button causes a progress bar of dots 7 and 8 to appear from left to right, after which you will see a flashing cursor and the name of the product. If VarioUltra is in braille notetaker mode you will be taken to the word processor entry in the main menu. If VarioUltra is in braille display mode, the display will appear blank when the device is ready.

Charging the Battery:
VarioUltra has an internal battery that is charged via the micro USB to standard type A USB cable. Note that the micro USB end of the cable that inserts into VarioUltra has a connector that is longer on one side than the other. The longer edge has two bumps on the surface, and needs to be on top in order to fit in VarioUltra. Forcing an upside down cable in to VarioUltra may break the USB port. Connect the other end of the cable into a USB plug or power adaptor (minimum rated 1A), or into the USB port of a computer. Note that charging via a computer will take longer to complete. To obtain battery status information in Braille Notetaker mode, give two short taps of the system key S3. Press the left thumb key B9 and system key S3 together to get the battery status in Braille Display mode.

NOTE: If the battery is allowed to drain completely, plugging VarioUltra to an external power source will not immediately bring it back to life. Please wait a few minutes for VarioUltra to attain a minimum amount of power, after which it will power on automatically, emitting a short beep as it does so.

Main Menu:
The braille notetaker features a main menu and sub menus. The main menu is where you will find applications. For now, these include word processor, pdf viewer, spreadsheet viewer, calculator, timers and alarms, file manager, settings, and help. Use display keys d1 and d3 to scroll up and down in the menu, or use the Navistick. Select an item by pressing the left thumb key B9. To return to the main menu at any time, just press and hold system key s3.

VarioUltra Commands:
Global shortcuts are VarioUltra commands that you can enter from anywhere. They always use the left thumb key, b9, along with other braille keys. Hotkeys are letters you enter in the menus to open an item. Global shortcuts and hotkeys are displayed for all menu items. The word processor item, for example, shows “MI” for menu item, the hotkey “w”, the item name “word processor”, and the global shortcut w+b8b9. If you are on the item in the menu, tapping b9 or pressing in on the Navistick will open it. To find out the shortcut command for other applications, scroll through the menu and read along each line.

System Keys:
Here are some quick tips for the system keys when in notetaker mode. For s1, press and hold to enter the word processor. S2 has no function. For s3, press and hold to return to the menu; double tap for battery level. For s4, tap once for current time, or twice for the current date.

Help:
Context Sensitive Help is available throughout VarioUltra, and provides information about an application and its functions. You can access Help by entering b9+b2b6 in Braille Notetaker mode. Use d2 and d5 to navigate left and right through the help messages. We encourage you to read the manual. If anything is unclear, let us know.

If you need technical support, please call us at: +49 (0) 3641/2816-0 or send an email to service@visiobraille.de or info@varioultra.com